PERCEPT Ltd   Drótos u. 12.,  H-1031 Budapest   HUNGARY

Tel./Fax.: +36 1 388 48 10

RAILWAY SIGNALS and "LSzR-x" TYPE SEMICONDUCTOR LIGHT SOURCES OF

WARNING LIGHTS

"LSzR" TYPE SEMICONDUCTOR LIGHT SOURCE FOR SIGNALling AND SAFETY DEVICES

This light source with LEDs is perfect for signalling and safety devices because

- it has pure (monochrome) colour (red 650 nm ±20nm, yellow 590 nm ±15nm, green 500 nm ±10nm, blue 470 nm ±10nm, white x=.31; y=.31 coord.)

- it has no phantom light

- it has intensive and constant brightness

- it is very quick	(Fmod > 1 MHertz )

- it has a very long life time

- it is very reliable	(MTBF more than 4 years)

- it is robustly built	(acceleration proof even above 10 gs)

- it has an almost perfect light characteristic	(can be asymmetric)

- it has good efficiency

and it doesn’t need any supplements such as lenses, colour filters, light bulbs or any other parts.
It can be easily fitted to electronic signal sources and devices.

Our product was optically qualified by the Magyar Elektrotechnikai Ellenörző Intézet (Hungarian Electrotechnical Controlling Institute), safety and railway qualification was made out by Budapesti Műszaki Egyetemen (Technical University of Budapest). Number of Certification is 485.005/1995.

## Table of currently available types

| Nominal size of installing bore in diameter | LSzR 40 Ø 40 mm | LSzR 150 Ø 150 mm | LSzR 200 Ø 200 mm |
| --- | --- | --- | --- |
| Diameter of fitting rim (approx.) | Ø 65 mm | Ø 165-170 mm | Ø 210-220 mm |
| Brightness | 25-60 candela | 120-750 candela | 150-1000 candela |
| Colour | red, yellow, green, blue, white | red, yellow, green, blue, white | red, yellow, green, blue, white |
| Nominal voltage of basic types | 24 V ±5% (D.C.) 12V±5% (D.C.) | 10,5 V± 15 % (DC , AC) 24 V± 15% (DC , AC) 35 V± 15% (DC, AC) | 10,5 V± 15 % (DC , AC) 24 V± 15% (DC , AC) 35 V± 15% (DC , AC) |
| Max. nominal input power | 1,4W without cooling, 3W with cooling | 17 W | 34 W |

For further information please contact the manufacturer:

Percept Ltd.	H-1031 Budapest, Drótos u. 12.	HUNGARY

# Tel., Fax:	+36-1-3884-810

E-mail:	percept@hu.inter.net; percept@t-online.hu

Lighting signals

#### Supply voltage:	51 V —77 V

Nominal voltage of the light source:	35 V

| Type (View angle) | ##### Colour | Nominal diameter | Voltage, Current | Light intensity (candela) |
| --- | --- | --- | --- | --- |
| LSzR-150-dj-8 degree LSzR-150-dj-8 degree LSzR-150-dj-8 degree | Red Yellow Green | 150 mm | 330 mA DC 35V  5% | 350-700 350-600 400-700 |
| LSzR-150-dj-20 degree LSzR-150-dj-20 degree LSzR-150-dj-20 degree LSzR-150-dj-20 degree | Red Yellow Green White | 150 mm | 330 mA DC 35V  5% | 350-450 350-400 350-500 250-350 |
| LSzR-150-dj-30 degree LSzR-150-dj-30 degree LSzR-150-dj-30 degree LSzR-150-dj-30 degree | Red Yellow Green White | 150 mm | 330 mA DC 35V  5% | 200-350 200-350 200-400 150-300 |
| LSzR-150-dja Shape signal | Green | 150 mm | 330 mA DC 35V  5% | 180-400 |
| LSzR-150-dt Reverse signal | Blue | 150 mm | 330 mA DC 35V  5% | 110-400 |

Blue reverse signal of 20 and 30 degree can be ordered as well.

Supply voltage:	24 V

Nominal voltage of the light source:	21 V

| Type (View angle) | ##### Colour | Nominal diameter | Voltage, Current | Light intensity (candela) |
| --- | --- | --- | --- | --- |
| LSzR-150-bj-8 degree LSzR-150-bj-8 degree LSzR-150-bj-8 degree | Red Yellow Green | 150 mm | 550 mA DC 21V  5% | 350-700 350-600 400-700 |
| LSzR-150-bj-20 degree LSzR-150-bj-20 degree LSzR-150-bj-20 degree LSzR-150-bj-20 degree | Red Yellow Green White | 150 mm | 550 mA DC 21V  5% | 350-450 350-400 350-500 250-350 |
| LSzR-150-bj-30 degree LSzR-150-bj-30 degree LSzR-150-bj-30 degree LSzR-150-bj-30 degree | Red Yellow Green White | 150 mm | 550 mA DC 21V  5% | 200-350 200-350 200-400 150-300 |
| LSzR-150-bja Shape signal | Green | 150 mm | 550 mA DC 21V  5% | 180-400 |
| LSzR-150-bt Reverse signal | Blue | 150 mm | 550 mA DC 21V  5% | 110-400 |

Blue reverse signal of 20 and 30 degree can be ordered as well.

Supply voltage:	12 V or 24 V

Nominal voltage of the light source:	10 V

| Type (View angle) | ##### Colour | Nominal diameter | Voltage, Current | Light intensity (candela) |
| --- | --- | --- | --- | --- |
| LSzR-150-cj-8 degree LSzR-150-cj-8 degree LSzR-150-cj-8 degree | Red Yellow Green | 150 mm | 1 A DC 10 V  5% | 350-700 350-600 400-700 |
| LSzR-150-cj-20 degree LSzR-150-cj-20 degree ###### LSzR-150-cj-20 degree LSzR-150-cj-20 degree | Red Yellow Green White | 150 mm | 1 A DC 10V  5% | 350-450 350-400 350-500 250-350 |
| LSzR-150-cj-30 degree LSzR-150-cj-30 degree LSzR-150-cj-30 degree LSzR-150-cj-30 degree | Red Yellow Green White | 150 mm | 1 A DC 10 V  5% | 200-350 200-350 200-400 150-300 |
| LSzR-150-ct Reverse signal | Blue | 150 mm | 1 A DC 10 V  5% | 110-400 |

Highlighted types are regularly available, the production period of the types in small letters is longer by two weeks.

Blue reverse signal of 20 and 30 degree can be ordered as well.

### Spark-free type

LSzR-150-rb type is developed for inflammable and explosive settings.
It must be used with proper, non-explosive circuit and fittings!

This light source is classified as IIB according to MSZ EN 50020:1992 Hungarian standard.

| Type (View angle) | ##### Colour | Nominal diameter | Voltage, Current | Light intensity (candela) |
| --- | --- | --- | --- | --- |
| LSzR-150-rb-8 degree LSzR-150-rb-8 degree LSzR-150-rb-8 degree | Red Yellow Green | 150 mm | 500 mA DC 9V  5% | 350-500 350-500 350-500 |
| LSzR-150-rb-20 degree LSzR-150-rb-20 degree LSzR-150-rb-20 degree LSzR-150-rb-20 degree | Red Yellow Green White | 150 mm | 500 mA DC 9V  5% | 300-400 300-400 300-400 250-350 |
| LSzR-150-rb-30 degree LSzR-150-rb-30 degree LSzR-150-rb-30 degree LSzR-150-rb-30 degree | Red Yellow Green White | 150 mm | 500 mA DC 9V  5% | 200-350 200-350 200-400 150-300 |
| LSzR-150-rba Shape signal | Green | 150 mm | 500 mA DC 9V  5% | 180-400 |
| LSzR-150-rbt-30 degree Reverse signal | Blue | 150 mm | 500 mA DC 9V  5% | 110-400 |

These types of light sources must not be put in circuit without external current restrictor, not even for a short time!